Name Surname: Furkan Özer						Date:05/12/2025

Number:2023555067									Report No:06

Digital Design Lab Report

Homework / Lab

Parts of Report:

- Description: Design an 8-bit register with synchronous reset, load control, and rotate-right operation.

- Turth Table of Design

| RESET | LOAD | ACTION | REG NEXT ST. |
| --- | --- | --- | --- |
| 1 | X | Reset | 0 |
| 0 | 1 | Parallel Load | din |
| 0 | 0 | Rotate Right | Reg(0)& Reg(1 to 7) |

- Drawing the circuit of desired desing

- Steps of Lab/Homework:

library IEEE;

use IEEE.std_logic_1164.all;

entity sgate is

port(

clk  : in std_logic;

rst  : in std_logic;

load : in std_logic;

din  : in std_logic_vector(7 downto 0);

q    : out std_logic_vector(7 downto 0)

);

end entity sgate;

architecture rtl of sgate is

signal reg : std_logic_vector(7 downto 0);

begin

process(clk)

begin

if rising_edge(clk) then

if (rst = '1') then

reg <= (others => '0');

elsif (load = '1') then

reg <= din;

else

reg <= reg(0) & reg(7 downto 1);

end if;        -- biten iç if (rst / load / else)

end if;            -- biten rising_edge if

end process;

q <= reg;

end architecture rtl;

Simulation Results:

Conclusion: The developed 8-bit register module, named sgate, operates in synchronization with the clock signal. The circuit's priority control structure ensures that the RST input clears all outputs first. In the absence of a reset condition, the circuit uses the LOAD signal to perform a parallel update with the DIN data. When neither of the higher-priority commands is active, the register maintains its contents in a circular right shift mode. The simulation results clearly validate this sequential behavior and confirm the successful verification of the designed multi-modal operations.