Name Surname:Furkan Özer							Date:19/12/2025

Number:2023555067								Report No:07

Digital Design Lab Report

Homework / Lab

- Description: Design an 8-bit synchronous Parity Check circuit using VHDL codes. Design in FOR-LOOP and WHILE-LOOP.

- Truth Table of Design

| Count of 1’s in i_data | Output |
| --- | --- |
| Even | 0 |
| Odd | 1 |

- Drawing the circuit of desired desing

- Steps of Lab/Homework:

| library IEEE; use IEEE.std_logic_1164.all; use IEEE.numeric_std.all; entity parity_check is port( i_clk : in std_logic; i_data : in std_logic_vector(7 downto 0); o_parity : out std_logic); end parity_check; architecture rtl of parity_check is signal r_data : std_logic_vector(7 downto 0); begin p_parity_check : process (i_clk) variable vparity : std_logic; variable v_index : integer; begin if rising_edge(i_clk) then r_data <= i_data; vparity := '0'; v_index := 0; while v_index < r_data'length loop vparity := vparity xor r_data(v_index); v_index := v_index + 1; end loop; o_parity <= vparity; end if; end process p_parity_check; end rtl; | library IEEE; use IEEE.std_logic_1164.all; use IEEE.numeric_std.all; entity parity_check is port( i_clk : in std_logic; i_data : in std_logic_vector(7 downto 0); o_parity : out std_logic); end parity_check; architecture rtl of parity_check is signal r_data : std_logic_vector(7 downto 0); begin p_parity_check : process (i_clk) variable vparity : std_logic; begin if rising_edge(i_clk) then r_data <= i_data; vparity := '0'; l_parity : for k in 0 to r_data'length-1 loop vparity := vparity xor r_data(k); end loop l_parity; o_parity <= vparity; end if; end process p_parity_check; end rtl; |
| --- | --- |

- Simulation Results:

- Conclusion: The proposed module functions synchronously with the clock signal. The input data (i_data) is stored using an internal signal named r_data. Within the process, a for/while loop scans through the bits of the registered data to compute the XOR result (parity). Simulation outcomes verify that the o_parity output accurately reflects whether the number of logic ‘1’s in the stored data is odd (’1’) or even (’0’).