Name Surname:Furkan Özer							Date:31/10/2025

Number:2023555067								Report No:02

Digital Design Lab Report

Homework / Lab

Parts of Report:

- Description: Design a circuit for given function with VHDL codes. Simplify the circuit before design. Explain how to make compilation and simulation. Show the simulation results.

Y= A’BC+ABD’+AB’C

Let’s simplify it step by step;

Y=A′BC+AB′C+ABD′

Y=C(A′B+AB′)+ABD′

We have to simplify inside the paranthesis

A’B+AB’

A′B+AB′=A⊕B That is the final simplified answer.

- Truth Table of Design

| A(input) | B(input) | C(input) | D(input) | Y(output) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 |

- Drawing the circuit of desired desing

- Steps of Lab/Homework:

library IEEE;

use IEEE.std_logic_1164.all;

entity xor_gate is

Port (

A : in  STD_LOGIC;

B : in  STD_LOGIC;

C : in  STD_LOGIC;

D : in  STD_LOGIC;

Y : out STD_LOGIC

);

end xor_gate;

architecture mimari of xor_gate is

begin

Y <= (C and (A xor B)) or (A and B and (not D));

end mimari;

- Simulation Results:

Conclution: After implementing the simplified expression Y = C(A ⊕ B) + ABD’ in VHDL and verifying it through simulation, the obtained truth table matches the expected one derived from the original Boolean expression Y = A’BC + ABD’ + AB’C. Therefore, the desired circuit is correctly obtained. The output values from the simulation confirm that the VHDL implementation behaves exactly as intended, producing the same results for all input combinations.